🚗 Smart Accident Detection & Emergency Alert System

1️⃣ Project Overview

📌 Problem Statement

- Road accidents cause millions of deaths annually

- Major issue: delay in emergency response

- Victims may be unconscious → unable to call for help ❌

✅ Solution

- Detect accident using MPU6050 (acceleration + tilt)

- Fetch location via GPS (NEO-6M)

- Send alert using WiFi (ESP32) or GSM

🌍 Real-world Applications

- 🚑 Emergency systems

- 🚗 Smart vehicles

- 📦 Fleet tracking

- 🛵 Bike safety systems

2️⃣ Components Required 💡

| Component | Purpose | Approx Cost (₹) |
| --- | --- | --- |
| ESP32 | Main controller | 300–500 |
| MPU6050 | Detect impact/tilt | 150 |
| NEO-6M GPS | Get location | 250–350 |
| GSM Module (SIM800L) (optional) | SMS alerts | 300 |
| Buzzer | Alert sound | 30 |
| Push Button | Cancel alert | 10 |
| Battery + Wires | Power supply | 200 |

👉 Total Budget: ~ ₹1000–1500 ✅

3️⃣ Circuit Diagram 🔌

🧠 Connections Overview

📍 MPU6050 → ESP32

- VCC → 3.3V

- GND → GND

- SDA → GPIO 21

- SCL → GPIO 22

📍 GPS (NEO-6M)

- VCC → 3.3V

- GND → GND

- TX → GPIO 16

- RX → GPIO 17

📍 Buzzer

  - → GPIO 25

  - → GND

📍 Button

- One side → GPIO 26

- Other → GND

📷 Visual Idea

4

4️⃣ Working Principle ⚙️

🔄 Step-by-step Flow

- ESP32 reads acceleration from MPU6050

- Detects sudden impact using threshold

- Checks tilt angle (vehicle flipped?)

- If threshold exceeded:

  - Trigger buzzer 🚨

  - Wait few seconds for cancel button

- If not cancelled:

  - Get GPS location 📍

  - Send alert message 📩

🎯 Threshold Logic (Important)

If acceleration > 2.5g OR tilt angle > 60°
→ Accident detected

6️⃣ Expected Output & Demo 📱

📩 Example Message:

🚨 Accident Detected!
📍 Location: https://maps.google.com/?q=13.0827,80.2707

🔥 What Happens:

- Sudden impact → buzzer ON

- No cancel → alert sent

- GPS link opens exact location

7️⃣ Enhancements 🚀

✅ Must Add:

- 🔘 Cancel button (avoid false alerts)

- 🔊 Buzzer alert

🌐 Advanced:

- Use ThingSpeak for dashboard

- Use Blynk mobile app

🤖 AI Idea (IMPRESSIVE)

- Train model on:

  - Speed

  - Impact force

- Predict:

  - Minor / Severe accident